# General

## Section includes

Detailed information for the preparation, submission, and Consultant’s review of operations and maintenance (O&M) data, as required by individual Specification sections.

## Definitions

Preliminary Data: Initial and subsequent submissions for Consultant’s review.

Final Data: Consultant-accepted data submitted as specified herein.

Maintenance Operation: As used on Maintenance Summary Form is defined to mean any routine operation required to ensure satisfactory performance and longevity of equipment. Examples of typical maintenance operations include but are not limited to lubrication, belt tensioning, adjustment of pump packing glands, and routine adjustments.

Instructional Manual: An organized compilation of operating and maintenance data including detailed technical information, documents and records describing operation and maintenance of individual systems, subsystems and components as specified in individual sections of this specification.

## Data format

Instructional Manual Hard Copy Format:

- Binder: Commercial quality, permanent, three metal post binders with durable plastic cover. Hot foil stamping on the cover and spine with applicable contract number and project description information. Model: Jay Weathercraft Ltd. (Toronto) post metal biner or equivalent.

Three-hole punch data for binding and composition; arrange printing so that punched holes do not obliterate data

Size: 8-1/2 inches by 11 inches, minimum.

Cover: Identify manual with typed or printed title “OPERATION AND MAINTENANCE DATA, VOLUME NO. ___ OF ___”,” and list:

- Project title.

Contractor’s name, address, and telephone number.

If entire volume covers equipment or system provided by one Supplier include the following:

Identity of general subject matter covered in manual.

Identity of equipment number and Specification section.

Provide each volume with title page and typed table of contents with consecutive page numbers. Place contents of entire set, identified by volume number, in each binder.

Table of contents neatly typewritten, arranged in a systematic order:

- Include list of each product, indexed to content of each volume.

Designate system or equipment for which it is intended.

Identify each product using the product name and other identifying numbers or symbols as set forth in Contract Documents.

Section Dividers:

- Heavy, 80-pound cover weight, tabbed with numbered plastic index tabs.

Fly-Leaf:

- For each separate product, or each piece of operating equipment, with typed description of product and major component parts of equipment.

List with each product:

Name, address, and telephone number of Subcontractor, Supplier, installer, and maintenance contractor, as appropriate.

Identify area of responsibility of each.

Provide local source of supply for parts and replacement.

Identity of separate structure as applicable.

Maintenance Summary (Format in accordance with paragraph 1.6.4)

Assemble and bind material in same order as specified in the Contract Documents.

Material shall be suitable for reproduction, with quality equal to original.

Electronic Media Format:

- Adobe Portable Document Format (PDF):

- After all preliminary data has been found to be acceptable to the Consultant, submit O&M data in PDF format electronically.

Files to be exact duplicates of Owner-accepted preliminary data. Arrange by specification number and name.

Files to be fully functional and viewable in most recent version of Adobe Acrobat.

PDF files to be indexed and searchable.

Manufacturers’ standard electronic format.

## Sequencing and scheduling

Equipment and System Data:

- Preliminary Data:

- Do not submit until Shop Drawing for equipment or system has been reviewed and accepted by the Consultant.

Submit prior to shipment date.

Final Data: Submit Instructional Manual Formatted data not less than 15 working days prior to equipment or system field functional testing. Submit compilation of Instructional Manual Formatted and Electronic Media Formatted data prior to Substantial Performance of Project.

Materials and Finishes Data:

- Preliminary Data: Submit at least 15 working days prior to request for final inspection.

Final Data: Submit within 10 days after final inspection.

## Submittals

Informational:

- Data Outline: Submit two copies of a detailed outline of proposed organization and contents of final data prior to preparation of preliminary data.

Preliminary Data:

- Submit electronically for Owner’s review.

If data meets conditions of the Contract, submit two hard copies.

- One copy will be returned to Contractor.

One copy will be forwarded to the Owner.

If data does not meet conditions of the Contract:

- Data will be returned to Contractor with Consultant’s comments (on separate document) for revision.

Consultant’s comments will be retained in Consultant’s file.

Resubmit the revised data electronically in accordance with Consultant’s comments.

Final Data: Submit two copies in format(s) specified herein.

## Data for systems, subsystem and components

Content For Each Unit (or Common Units) and System:

- Product Data:

- Include only those sheets that are pertinent to specific product.

Clearly annotate each sheet to:

- Identify specific product(s) or part(s) installed.

Identify data applicable to installation.

Delete references to inapplicable information.

Function, normal operating characteristics, and limiting conditions.

Serial Numbers

Performance curves, engineering data, nameplate data, and tests reports for all pumps.

Complete nomenclature and commercial number of replaceable parts.

Original manufacturer’s parts list, illustrations, detailed assembly drawings showing each part with part numbers and sequentially numbered parts list, and diagrams required for maintenance.

Spare parts ordering instructions.

Where applicable, identify installed spares and other provisions for future work (e.g., reserved panel space, unused components, wiring, terminals).

As-installed, color-coded piping diagrams.

Charts of valve tag numbers, with the location and function of each valve.

Drawings: Supplement product data with Drawings as necessary to clearly illustrate:

- Format:

- Provide reinforced, punched, binder tab; bind in with text.

Reduced to 8-1/2 inches by 11 inches, or 11 inches by 17 inches folded to 8-1/2 inches by 11 inches.

Where reduction is impractical, fold and place in 8-1/2-inch by 11-inch envelopes bound in text.

Identify Specification section and product on Drawings and envelopes.

Relations of component parts of equipment and systems.

Control and flow diagrams.

Coordinate drawings with Project record documents to assure correct illustration of completed installation.

Instructions and Procedures: Within text, as required to supplement product data.

- Format:

- Organize in consistent format under separate heading for each different procedure.

Provide logical sequence of instructions for each procedure.

Provide information sheet for Owner’s personnel, including:

- Proper procedures in event of failure.

Instances that might affect validity of warranty.

Installation Instructions: Including alignment, adjusting, calibrating, and checking.

Operating Procedures:

- Startup, break-in, routine, and normal operating instructions.

Test procedures and results of factory tests where specified.

Regulation, control, stopping, and emergency instructions.

Description of operation sequence by control manufacturer.

Shutdown instructions for both short and extended duration.

Summer and winter operating instructions, as applicable.

Safety precautions.

Special operating instructions.

Maintenance and Overhaul Procedures:

- Routine maintenance.

Guide to troubleshooting.

Disassembly, removal, repair, reinstallation, and re-assembly.

Warranty and Service Agreement: In accordance with Section 01 77 00, Closeout Procedures and Section 01 78 36 – Extended Warranties.

Content for Each Electric or Electronic Item or System:

- Description of Unit and Component Parts:

- Function, normal operating characteristics, and limiting conditions.

Performance curves, engineering data, nameplate data, and tests.

Complete nomenclature and manufacturer’s part number for replaceable parts.

Interconnection wiring diagrams, including control and lighting systems.

Circuit Directories of Panel boards:

- Electrical service.

Controls.

Communications.

List of electrical relay settings, and control and alarm contact settings.

Electrical interconnection wiring diagram, including control and lighting systems.

As-installed control diagrams by control manufacturer.

Operating Procedures:

- Routine and normal operating instructions.

Sequences required.

Safety precautions.

Special operating instructions.

Maintenance Procedures:

- Routine maintenance.

Guide to troubleshooting.

Adjustment and checking.

List of relay settings, control and alarm contact settings.

Manufacturer’s printed operating and maintenance instructions.

List of original manufacturer’s spare parts, manufacturer’s current prices, and recommended quantities to be maintained in storage.

Content for Programmable Devices/Components/Sub-systems:

- The following requirements are minimum requirements applicable to programmable equipment such as Variable Frequency Drive (VFD), microprocessor-based devices, Programmable Logic Controllers (PLC), Human-Machine-Interfaces (HMI), computers, and other programmable devices. Additional requirements may be specified elsewhere.

- As-Constructed version of shop drawings.

Functional description.

Wiring details.

Configuration Records; record of switch settings, program listings and parameter settings, after commissioning.

Maintenance manuals.

User guides, technical reference and programming manuals.

Electronic copies of:

- Manuals.

Settings, databases and templates. Include both native format of files and ASCII-exported version.

Application programs.

Cable and software for use on Owner’s portable computer for revising/downloading the settings and software.

Maintenance Summary:

- Compile individual Maintenance Summary for each applicable equipment item, respective unit or system, and for components or sub-units.

Format:

- Use Maintenance Summary Form bound with this section or electronic facsimile of such.

Each Maintenance Summary may take as many pages as required.

Use only 8-1/2-inch by 11-inch size paper.

Complete in typewritten format.

Include detailed lubrication instructions and diagrams showing points to be greased or oiled; recommend type, grade, and temperature range of lubricants and frequency of lubrication.

Recommended Spare Parts:

- Data to be consistent with manufacturer’s bill of materials/parts list furnished in O&M manuals.

“Unit” is the unit of measure for ordering the part.

“Quantity” is the number of units recommended.

“Unit Cost” is the current purchase price.

## Data for materials and finishes

Content for Architectural Products, Applied Materials, and Finishes:

- Manufacturer’s data, giving full information on products:

- Catalog number, size, and composition.

Color and texture designations.

Information required for reordering special-manufactured products.

Instructions for Care and Maintenance:

- Manufacturer’s recommendation for types of cleaning agents and methods.

Cautions against cleaning agents and methods that are detrimental to product.

Recommended schedule for cleaning and maintenance.

Content for Moisture Protection and Weather Exposed Products:

- Manufacturer’s data, giving full information on products:

- Applicable standards.

Chemical composition.

Details of installation.

Instructions for inspection, maintenance, and repair.

## Supplements

The supplements listed below, following “End of Section”, are part of this Specification.

- Forms: Maintenance Summary Form.

# Products – Not used

# Execution – Not used

End of section

MAINTENANCE SUMMARY FORM

PROJECT:	 CONTRACT NO.:

1. EQUIPMENT ITEM

2. MANUFACTURER

3. MODEL

4. SERIAL NUMBER

5. EQUIPMENT/TAG NUMBER(S)

6. WEIGHT OF INDIVIDUAL COMPONENTS (OVER 45 KG)

7. NAMEPLATE DATA (hp, voltage, speed, etc.)

8. MANUFACTURER’S LOCAL REPRESENTATIVE

a.	Name	 Telephone No.

b.	Address

9. MAINTENANCE REQUIREMENTS

| MAINTENANCE OPERATION COMMENTS | FREQUENCY | LUBRICANT (IF APPLICABLE) |
| --- | --- | --- |
| List briefly each maintenance operation required and refer to specific information in manufacturer’s standard maintenance manual, if applicable. (Reference to manufacturer’s catalog or sales literature is not acceptable.) | List required frequency of each maintenance operation. | Refer by symbol to lubricant required. |

10. LUBRICANT LIST

| REFERENCE SYMBOL | SHELL | STANDARD OIL | GULF | ARCO | OR EQUAL |
| --- | --- | --- | --- | --- | --- |
| List symbols used in No. 9. above. | List equivalent lubricants, as distributed by each manufacturer for the specific use recommended. |  |  |  |  |

11. RECOMMENDED SPARE PARTS FOR OWNER’S INVENTORY.

| PART NO. | DESCRIPTION | UNIT | QUANTITY | UNIT COST |
| --- | --- | --- | --- | --- |
| Note: Identify parts provided by this Contract with two asterisks. |  |  |  |  |