# General

## Definitions

Action Submittal: Written and graphic information submitted by Contractor, that requires Consultant’s review.

Informational Submittal: Documents submitted for Information by Contractor that are factual in nature and do not require Consultant’s review. Submittals not meeting conditions of the Contract will be returned.

## Procedures

Provide submittals to the Consultant in electronic format unless specified otherwise.

- Each submittal shall be electronic file in Adobe Acrobat PDF. Use latest version available at time of execution of Agreement.

Electronic files that contain more than 10 pages in PDF format shall contain internal book marking from index page to major sections of document.

PDF files shall be set to open “Bookmarks and Page” view.

Add general information to each PDF file, including title, subject, author, and keywords.

PDF files shall be set up to print legibly on Letter size (8-1/2 inches by 11 inches) or Ledger/Tabloid size (11 inches by 17 inches) paper. No other paper sizes will be accepted.

Submit new electronic files for each resubmittal.

Include copy of Transmittal of Contractor’s Submittal form, located at end of section, with each electronic file.

Upload electronic submittals to file sharing site provided by the Consultant.

Submit authorization for the Consultant to reproduce and distribute each file as many times as necessary for project documentation.

Detailed procedures for handling electronic submittals will be discussed at Preconstruction meeting.

List of Submittals

- For each specification section submittal, the Contractor shall provide a table listing all the submittals anticipated for that specification section. The table shall include the following information:

Specification section

Total Number of Submittals for this section

Shop drawings associated with each submittal

Revision and status for each submittal

Transmittal of Submittal:

- Review each submittal and check for compliance with Contract Documents.

Stamp each submittal with uniform approval stamp before submitting to the Consultant.

Stamp to include Project name, submittal number, specification number, Contractor’s reviewer name, date of Contractor’s approval, and statement certifying that submittal has been reviewed, checked, and approved for compliance with Contract Documents.

Consultant will not review submittals that do not bear Contractor’s approval stamp and will return them without action.

Consultant will not review submittals received directly from a Subcontractor or Supplier and will return them without action.

Identify each submittal with the following:

- Numbering and Tracking System:

- Sequentially number each submittal.

Resubmission of submittal shall have original number with sequential alphabetic suffix.

Specification section and paragraph to which submittal applies.

Project title and Region of Durham project number: Example: Lake Simcoe WPCP Aeration Systems Upgrades T001397A

Date of transmittal.

Names of Contractor, Subcontractor or Supplier, and manufacturer as appropriate.

Identify and describe each deviation or variation from Contract Documents.

Include Contractor’s written response to each of Consultant’s review comments with resubmission of submittals stamped “Exceptions Noted, Resubmit”.

Submit Contractor’s written acknowledgement and acceptance of each of Consultant’s review comments on submittals stamped “Exceptions Noted”.

Format:

- Do not base Shop Drawings on reproductions of Contract Documents.

Package submittal information by individual specification section. Do not combine different specification sections together in submittal package, unless otherwise directed in specification.

Present in a clear and thorough manner and in sufficient detail to show kind, size, arrangement, and function of components, materials, and devices, and compliance with Contract Documents.

For any hard copy submittal, index with labeled tab dividers in orderly manner.

Timeliness:

- Schedule and submit in accordance with schedule of Shop Drawing and Sample submittals, and requirements of individual specification sections.

Submit Shop Drawings and Samples well in advance of scheduled delivery date for associated equipment or material.

Co-ordinate submittals prepared by multiple trades such that information is available to allow prior review and sufficient review time where work of one trade interfaces with or affects work of another.

Processing Time:

- Time for review shall commence on Consultant’s receipt of submittal.

Consultant will act upon Contractor’s submittal and transmit response to Contractor not later than ten (10) Working Days after receipt, unless otherwise specified and if Contractor is notified by the Consultant a longer review will be required. If additional review time is needed the Consultant will notify the Contractor within five Working Days of receipt of the submittal(s).

Resubmittals will be subject to same review time.

Allow additional review time for complex equipment and systems.

Resubmittals:

- Clearly identify each correction or change made and include revision date.

Provide clear response to each itemized comment by the Consultant on the submittal, whether or not action has been taken, and description of action.

No adjustment of Contract Times or Price will be allowed due to delays in progress of Work caused by rejection and subsequent resubmittals.

Note the Owner will charge the cost of Consultant’s review of submittals which have been submitted more than two times to the Contractor’s account.

Incomplete Submittals:

- Consultant will return entire submittal for Contractor’s revision if preliminary review deems it incomplete.

Submittal will be deemed incomplete if:

- Contractor’s review stamp is not completed and signed.

Transmittal of Contractor’s Submittal are not completed and signed.

All requested information is not provided.

Submittals missing Professional Consultant’s seal and signature, where it is required.

Insufficient number of copies.

Submittals not required by Contract Documents:

- Will not be reviewed and will be returned stamped “Not Subject to Review.”

Do not revise submittals after they have been reviewed and stamped “No Exceptions Taken” or “Exceptions Noted”.

Consultant will complete up to two reviews of each submittal at no cost to Contractor. Owner will deduct cost of additional reviews from Contract Price.

## Action submittals

General:

- Prepare and submit Action Submittals required by individual specification sections.

Consultant will review Action Submittals only for general conformance with design concept and general compliance with Contract Documents. Consultant’s review does not relieve Contractor from compliance with requirements of Contract Documents nor from errors in submittals or Contractor’s design.

Contractor is responsible for confirmation of dimensions at jobsite; fabrication processes; means, methods techniques, sequences and procedures of construction; co-ordination of Work of all trades; and performance of Work in safe and satisfactory manner.

Consultant’s review comments and review stamp will be placed electronically on submitted Shop Drawings or on separate submittal review comment form.

Shop Drawings:

- Copies: One electronic copy.

Identify and indicate:

- Applicable Contract Drawing and Detail number, products, units and assemblies, and system or equipment identification or tag numbers.

Include in every shop drawing submission, a copy of the relevant specification section, with addendum updates included, and all referenced and applicable sections, with addendum updates included. Checkmark each paragraph to indicate compliance with the specification or mark otherwise to indicate requested deviations from specified requirements.  Check marks () denote full compliance with a paragraph in its entirety.  If deviations from the specifications are indicated, underline each point of deviation and denote by a number in the margin to the right of the identified paragraph.  The remaining portions of the paragraph not underlined will signify compliance with the specified requirements.  Provide in the submittal a detailed, written justification for each deviation.

Equipment and Component Title: Identical to title shown on Drawings.

Critical field dimensions and relationships to other critical features of Work. Note dimensions established by field measurement.

Project-specific information drawn accurately to scale.

Shop Drawings for Temporary Works:

- Submit for review shop drawings of temporary works which:

- Control the dimensions and locations of any part of the structures to be constructed under the contract.

Impose loads on parts of the works which are still under construction or on existing structures.

Payment will not be made for work started or completed without the required drawing review. Submit shop drawings well in advance of the time when they are required for construction. Co-ordinate shop drawings prepared by different trades so that information is available to prevent conflict or errors where the work of one trade affects the work of another.

Shop drawings will be reviewed for general conformity with the required arrangement and dimensions of the permanent structures and for general conformity with the specifications.

If resubmittal is requested, discuss the comments made and resolve all issues raised by them, then resubmit the shop drawings amended accordingly.

Do not begin construction of temporary works until after the completion of review of the shop drawings.

Review of the Contractor’s drawings does not relieve the Contractor of the responsibility for the results arising from errors or emissions of design or from the use or abuse of the temporary work.

Keep one copy of each stamped, reviewed shop drawing at the site of the work for reference during the time the construction work is in progress.

Make no change in drawings after they have been reviewed.

Submit shop drawings in SI metric units.

Manufacturer’s standard schematic drawings and diagrams as follows:

- Modify to delete information that is not applicable to the Work.

Supplement standard information to provide information specifically applicable to the Work.

Product Data: Provide as specified in individual specifications.

Manufacturers: Include the following additional information:

- Names and addresses of technical service representatives close to project.

Complete list of spare parts and accessories for each piece of equipment, identifying whether the parts are kept in stock by the above service representatives.

Units: Submit all Shop Drawings in SI metric units.

Required submittals include but are not limited to:

- Catalogue Drawings: Include reprints of catalogue drawings of proprietary articles of standard fabrication and manufacture for the work.

Shop Drawings: Include dimensioned line drawings and related specifications, information and literature for custom fabricated articles and equipment.

ISA data sheets for all instruments.

Samples:

- Copies: Two, unless otherwise specified in individual specifications.

Preparation: Mount, display, or package Samples in manner specified to facilitate review of quality. Attach label on unexposed side that includes the following:

- Manufacturer name.

Model number.

Material.

Sample source.

Manufacturer’s Colour Chart: Units or sections of units showing full range of colours, textures, and patterns available.

Full-size Samples:

- Size as indicated in individual specification section.

Prepared from same materials to be used for the Work.

Cured and finished in manner specified.

Physically identical with product proposed for use.

Do not use materials in Work which are in any way inferior to Samples submitted and reviewed. Match accepted samples.

Review of samples notwithstanding, materials that are unsound or imperfect when delivered to site will be rejected.

Retain reviewed samples on site readily available to Consultant.

Action Submittal Dispositions: Consultant will review, mark, and stamp as appropriate, and distribute marked-up copies or submittal review comment forms as noted:

Update the following based on consultant's preference.

- No Exceptions Taken (NET):

- Contractor may incorporate product(s) or implement Work covered by submittal.

Exceptions Noted (EN):

- Contractor may incorporate product(s) or implement Work covered by submittal, in accordance with Consultant’s notations.

Exceptions Noted, Resubmit (ENR):

- Make corrections or obtain missing portions and resubmit.

Contractor may not incorporate product(s) or implement Work covered by submittal, except portions where indicated Contractor may begin to incorporate product(s) or implement Work covered by the submittal in accordance with the Consultant’s notations.

## Informational submittals

General:

- Copies: Submit electronically. If this is not possible, submit three hard copies, unless otherwise indicated in individual specification section.

Refer to individual specification sections for specific submittal requirements.

Where work is to be designed by Contractor, comply with applicable codes and submit Shop Drawings signed and sealed by professional licensed in province of work.

Consultant will review each submittal for general conformance with design intent and general compliance with Contract Documents. Consultant’s review does not relieve the Contractor from compliance with requirements of Contract documents nor from errors in the submittal or Contractor’s design.

Certificates:

- General:

- Provide notarized statement that includes signature of entity responsible for preparing certification.

Signed by officer or other individual authorized to sign documents on behalf of that entity.

Installer: Prepare written statements on manufacturer’s letterhead certifying that installer complies with requirements as specified in individual specification sections.

Material Test: Prepared by qualified testing agency, on testing agency’s standard form, indicating and interpreting test results of material for compliance with requirements.

Certificates of successful testing or inspection: Submit when testing or inspection is required by laws and regulations or governing agency or specified in individual specification sections.

Manufacturer’s Certificate of Compliance: In accordance with Section 01 43 33 – Manufacturer’s Field Services.

Manufacturer’s Certificate of Proper Installation: In accordance with Section 01 43 33 – Manufacturer’s Field Services.

Construction Photographs and Video: In accordance with Section 01 31 13 – Project Co-ordination, and as may otherwise be required in Contract Documents.

Contract Closeout Submittals: In accordance with Section 01 77 00 – Closeout Procedures.

Contractor’s Design Data:

- Written and graphic information.

List of assumptions.

List of performance and design criteria.

Summary of loads or load diagram, if applicable.

Calculations.

List of applicable codes and regulations.

Name and version of software.

Information requested in individual specification section.

Seal and signature of professional licensed in the province of the work.

Manufacturer’s Instructions: Written or published information that documents manufacturer’s recommendations, guidelines, and procedures in accordance with individual specification sections.

Operation and Maintenance Data: As required in Section 01 78 23 – Equipment Operation and Maintenance Data.

Schedules:

- Schedule of Shop Drawing and Sample Submittals: Prepare separately or in combination with progress schedule as specified in Section 01 32 16 – Construction Progress Schedule.

- Show for each, at a minimum, the following:

- Specification section number.

Identification by numbering and tracking system as specified under Paragraph Transmittal of Submittal.

Estimated date of submission to Consultant, including reviewing and processing time.

On a monthly basis, submit updated schedule to Consultant if changes have occurred or resubmittals are required.

Progress Schedules: In accordance with Section 01 32 16 – Construction Progress Schedule, 01 31 13 - Project Co-ordination, and 01 32 13 – Scheduling of the Work.

Special Warranty: Supplier’s written warranty as required in individual specification sections.

Statement of Qualification: Evidence of qualification, certification, or registration as required in Contract Documents to verify qualifications of professional land surveyor, engineer, materials testing laboratory, specialty Subcontractor, trade, specialist, consultant, installer, and other professionals.

Submittals Required by Laws, Regulations, and Governing Agencies:

- Submit promptly notifications, reports, certifications and otherwise as may be required, directly to the applicable federal, provincial, or local governing agency or their representative.

Transmit to Consultant for Owner’s records one copy of correspondence and transmittals (to include enclosures and attachments) between Contractor and governing agency.

Test and Inspection Reports:

- General:

- Shall contain signature of person responsible for test or report.

Complete an equipment report prior to the site testing each item of rotating mechanical equipment. During testing complete the remainder of the equipment report. A sample of the equipment report is included at the end of this section. Submit the reports for inclusion in the equipment operation and maintenance manual.

Before operating equipment, engage the services of a qualified manufacturer’s service representative to inspect, operate, test and adjust the equipment after installation.

Factory:

- Identification of product and specification section, type of inspection or test with referenced standard or code.

Date of test, Project title and number, and name and signature of authorized person.

Description of inspection, test results, adjustments made, quantitative results and suggestions for precautions to be taken for correct maintenance.

If test or inspection deems material or equipment not in compliance with Contract Documents, identify corrective action necessary to bring into compliance.

Provide interpretation of test results, when requested by the Consultant.

Verify that the equipment and its installation conform to the requirements of the Contract for the service intended and is ready for permanent operation. Submit copies of report for binding into the equipment operation and maintenance manuals.

Other items and requirements as identified in individual specification sections.

Field: As a minimum, include the following:

- Project title and number.

Date and time.

Record of temperature and weather conditions.

Identification of product and specification section.

Type and location of test, sample, or inspection, including referenced standard or code.

Date issued, testing laboratory name, address, and telephone number, and name and signature of laboratory inspector.

If test or inspection deems material or equipment not in compliance with Contract Documents, identify corrective action necessary to bring into compliance.

Provide interpretation of test results, when requested by the Consultant.

Other items as identified in individual specification sections.

Inspection includes:

- Soundness (without cracked or otherwise damaged parts).

Completeness of installation as specified and as recommended by manufacturer.

Correctness of setting, alignment, and relative arrangement of various parts of system.

Operate, test, and adjust equipment to prove it is correctly installed to operate under the intended conditions.

Equipment will only be accepted after receipt of the manufacturer’s representative’s report.

Submit notice in writing at least 48 hours before manufacturer’s representative is scheduled to perform these services.

Modify or replace equipment or materials failing required tests.

Perform additional testing required due to changes of materials requested by Contractor or due to failure of materials or construction to meet specifications.

Mandatory forms following this section include: Concrete Pour Release Form, Contractor Submittal Transmittal, Equipment Report Form, Extra Work Report, and Pressure Testing Report.

Maintenance Materials:

- Submit special tools, lubricants, maintenance materials, spare parts, manuals, as-built drawings, and reports to the Consultant.

Submissions to include a complete list of items delivered. Submissions must be acknowledged in writing as being received by the Consultant and the Owner’s representative.

Submit manuals a minimum of one week prior to commissioning of the equipment.

Testing and Startup Data: In accordance with Section 01 91 00 – Commissioning.

Training Data: In accordance with Section 01 43 33 – Manufacturer’s Field Services.

Interference Drawings:

- Before installation of structural, mechanical and electrical systems, prepare an integrated set of interference drawings in co-operation with all trades.

Show locations and relationship of all trades in mechanical and electrical equipment rooms, pipe spaces, tunnels, galleries and basements.

Show locations of all electrical conduits and pipes to be cast into structural slabs, walls and columns.

Submit updated interference drawings as work progresses.

Make necessary relocations due to interference between trades, as a result of incomplete interference drawings, at no cost to Owner.

As-Built Documents: In accordance with Section 01 77 00 – Closeout Procedures.

Spare Parts and Loose Items List:

- Submit recommended spare parts list together with the first shop drawings submittal for all designated equipment. The detailed specifications state which equipment requires recommended spare parts lists. Where not stated in specifications, provide manufacturer’s recommended list of spare parts. Reference specification section number for each designated equipment on spare parts list.

The recommended spare parts list must include, but not limited to:

- Item number.

Description of spare part or item.

Equipment vendor or supplier’s name.

Reference specification section (and clause, if applicable).

Quantity of item or part.

Lead time.

Date received.

Date turned over to Owner.

Price of each part at time of submittal and delivery.

Comments.

The recommended spare parts list will be used by the Owner as a guide for planning suitable spare parts inventory prior to acceptance of the equipment.

Submit updated spare parts list indicating spare parts handed over to the Owner.

Extra Materials List:

- Submit recommended extra materials list with first submittal of designated items. The detailed specification state which items require extra materials. Reference specification section number for each item listed.

The extra materials list must include, but not limited to:

- Item number.

Description of extra materials.

Extra material supplier name.

Reference specification section (and clause, if applicable).

Quantity of extra material.

Date received.

Date turned over to Owner.

Comments.

Submit updated extra materials list indicating items handed over to the Owner.

Equipment Control Log:

- Submit log listing equipment installed under Contract referencing applicable specification section for each piece of equipment.

Include in equipment control log:

- Equipment description.

Reference specification section (and clause, if applicable).

Equipment tag number.

Equipment Vendor or supplier.

Date required on site.

Date delivered to site.

Date installed.

O&M Manual submitted and accepted.

Training completed.

Pre-start health and safety review completed.

Startup date.

Comments.

Submit updates as equipment is installed, or if schedule dates change.

Information Submittal Dispositions:

- If submittal meets requirements of Contract, Consultant will forward copies to appropriate parties. Some copies will be marked “For Information Only” (FIO).

If Consultant determines that submittal does not meet requirements of Contract, submittal will be considered unacceptable. Consultant will retain one copy and return remaining copies with review comments to Contractor for correction and resubmission.

## Supplements

The supplements listed below, following “End of section”, are part of this specification.

- Forms: Transmittal of Contractor’s Submittal

# Products – Not used

# Execution – Not used

End of section

| TRANSMITTAL OF CONTRACTOR’S SUBMITTAL (ATTACH TO EACH SUBMITTAL) Date: |  |  |  |
| --- | --- | --- | --- |
| TO: FROM: Contractor |  | Submittal Number: New Submittal Resubmittal Project: Project Number: Specification Section Number: (Address only one specification section with each transmittal) Schedule Date of Submittal: |  |
| SUBMITTAL TYPE: | Shop Drawing | Sample | Informational |

The following items are hereby submitted:

| Number of Copies | Description of Item Submitted (Type, Size, Model Number, etc.) | Specification and Paragraph Number | Drawing or Brochure Number | Contains Variance to Contract |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | No | Yes |

Contractor hereby certifies that they have complied with the requirements of the Contract Documents in preparation, review, and submission of designated Submittal and that the Submittal is complete and in accordance with the Contract Documents and requirements of laws and regulations and governing agencies.

By:___________________________________

Contractor (Authorized Signature)